FUEL INJECTION

## SYNOPSIS

During the compression stroke in a four-stroke diesel engine, air is compressed in the engine cylinder.  The pressure of the air is increased and its temperature is also increased.  The diesel fuel is injected at the end of the compression stroke and the fuel is ignited.

The fuel feed system ensures that the diesel oil is injected into the cylinders at the correct time.  It consists of a diesel tank, a feed pump, a filter, an injection pump, an injector and connecting lines.  Regular testing of the fuel injection system ensures that the diesel pump works effectively.  Testing is carried out by:

- Calibration and

- Phasing

INTRODUCTION

To run an engine, the fuel from the tank must each by some means to the engine cylinder. In diesel engine, the fuel is injected into the engine cylinder by an injector. The fuel burns in the cylinder and during the exhaust stroke, the burned gases leave the cylinder passing through the exhaust pipe and silencer.

The injector tester consists of a small tank, pump, pressure gauge and handle.  There is a separate bowl for receiving the fuel sprayed from the nozzle.  The injector to be tested is fitted in the injection testing equipment.  A valve which is used to control the fuel is first opened, and then the handle is pressed downward.

The downward movement of the handle causes the fuel to be sprayed through the injector.  The reading in the pressure gauge shows the atmospheric pressure.  If this pressure is equal to the pressure specified by the manufacturer, then the injector is a good one.  If the pressure is either more or less, the spring in the injector should be accordingly adjusted.

ADVANTAGES

- Higher power.

- Low specific fuel consumption.

- Simplified induction manifolds free from strangled entry.

- No necessity to maintain a stable ignition mixture in the manifold.

- No necessity of induction heating.

- Free from icing trouble.

- Avoidance of acceleration disturbances during cornering and breaking.

- Quick starting and warm-up.

- Greater freedom in choice of fuels.

### DISADVANTAGES

- Higher initial cost.

- Higher maintenance cost.

- Complicated design.

- Difficult in operation.

- Difficult servicing.