| Title | Test, diagnose, and rectify faults in an electronic fuel injection (EFI) system on an engine |  |  |
| --- | --- | --- | --- |
| Level | 4 | Credits | 4 |

| Purpose | This unit standard is for people in the automotive industry. People credited with this unit standard are able to: test and diagnose faults in an EFI system on an engine; test and service the EFI fuel injectors; and rectify EFI system faults on an engine. |
| --- | --- |

| Classification | Motor Industry > Automotive Fuel Systems and Exhaust |
| --- | --- |

| Available grade | Achieved |
| --- | --- |

| Entry information |  |
| --- | --- |
| Critical health and safety prerequisites | Unit 899, Carry out automotive wiring and circuit repairs, or demonstrate equivalent knowledge and skills. |
| Recommended skills and knowledge | Unit 5459, Explain the operation of automotive electronic fuel injection systems; and Unit 5463, Explain the operation and repair requirements of automotive electronic control systems; or demonstrate equivalent knowledge and skills. |

Explanatory notes

1	The following legislation and amendments are required to be consulted and followed where applicable:

–	Health and Safety in Employment Act, 1992.

2	Reference to suitable tools and equipment means industry approved tools and equipment that are recognised within the industry as being the most suited to complete the task to a professional and competent manner with due regard to safe working practices.

3	Because of the particular nature of this unit standard, it is essential that the practical assessment evidence is obtained in the workplace under normal workplace conditions.

Outcomes and evidence requirements

Outcome 1

Test and diagnose faults in an EFI system on an engine.

Evidence requirements

1.1	Safe working practices are observed throughout the task.

Range	personal safety; safety of others; equipment, vehicle, and machine safety.

1.2	The fuel system and the engine compartment are checked visually to ensure no obvious faults exist.

Range	loose and leaking fuel and coolant pipes and hoses, loose and disconnected wires and connectors, low fluid levels, loose and missing drive belts, any other loose and broken components.

1.3	Suitable equipment is selected and used to enable system tests to be completed.

1.4	No damage is caused to circuit components and electronic devices when using test instruments.

1.5	The self-test codes are activated, read, and interpreted to identify any circuit that has a fault.

1.6	The circuit indicated by the self-test codes is tested, and the fault is located and identified.

1.7	EFI circuits not monitored by the self-test system are tested, and any faults are located and identified.

1.8	The air intake system is checked for blockages and leaks and to determine if it is operating correctly.

Range	air flow sensor, trunking, throttle body, auxiliary air valve, vacuum hoses, intake manifold, air filter.

1.9	The fuel supply system is tested for operating pressure, pressure retention, and fuel flow rate, and the results are noted.

1.10	The exhaust emissions and engine speed are measured at normal operating temperature, and the results are noted.

1.11	The results of the above tests are noted and compared with the vehicle specification, and are analysed to locate and identify the cause of any fault.

Outcome 2

Test and service the EFI fuel injectors.

Evidence requirements

2.1	Safe working practices are observed throughout the task.

Range	personal safety; safety of others; equipment, vehicle, and machine safety.

2.2	Suitable tools and equipment are selected and used to enable tests and the service work to be carried out.

2.3	The injectors are tested to determine which are unserviceable and the nature of any unserviceability.

Range	tests – noise of operation, pulse to injectors, revolutions per minute drop when injectors are disconnected, internal resistance, spray pattern, atomisation, fuel throughput, leakage.

2.4	The injectors are cleaned to restore normal operation by running the engine on a chemical cleaning fuel.

2.5	Reject injectors and injector seals are replaced with new ones that meet manufacturer's specifications.

Outcome 3

Rectify EFI system faults on an engine.

Evidence requirements

3.1	Safe working practices are observed throughout the task.

Range	personal safety; safety of others; equipment, vehicle, and machine safety.

3.2	Any faulty EFI system components are replaced with parts that meet manufacturer's specifications, and in a manner prescribed by the manufacturer.

3.3	The system is tested to ensure the integrity of the wiring is maintained in a serviceable condition.

3.4	The system is tested to ensure that there are no fuel blockages.

3.5	The system is tested to ensure that there are no air or fuel leaks.

3.6	All adjustments are checked and where necessary, are corrected to comply with manufacturer's specifications.

Range	throttle pedal travel and the pedal stop, throttle body settings, idle-up devices, sensor switches, engine idle speed, mixture strength.

3.7	The EFI system operates and performs to manufacturer's specifications.

| Replacement information | This unit standard, unit standard 961, and unit standard 5460 have been replaced by unit standard 24090 and unit standard 24091. |
| --- | --- |

This unit standard is expiring.  Assessment against the standard must take place by the last date for assessment set out below.

Status information and last date for assessment for superseded versions

| Process | Version | Date | Last Date for Assessment |
| --- | --- | --- | --- |
| Registration | 1 | 29 October 1993 | 31 December 2016 |
| Review | 2 | 4 October 1996 | 31 December 2016 |
| Review | 3 | 26 February 1999 | 31 December 2016 |
| Review | 4 | 25 January 2008 | 31 December 2016 |
| Rollover | 5 | 19 November 2010 | 31 December 2016 |

| Accreditation and Moderation Action Plan (AMAP) reference | 0014 |
| --- | --- |

This AMAP can be accessed at http://www.nzqa.govt.nz/framework/search/index.do.

Please note

Providers must be granted consent to assess against standards (accredited) by NZQA, or an inter-institutional body with delegated authority for quality assurance, before they can report credits from assessment against unit standards or deliver courses of study leading to that assessment.

Industry Training Organisations must be granted consent to assess against standards by NZQA before they can register credits from assessment against unit standards.

Providers and Industry Training Organisations, which have been granted consent and which are assessing against unit standards must engage with the moderation system that applies to those standards.

Consent requirements and an outline of the moderation system that applies to this standard are outlined in the Accreditation and Moderation Action Plan (AMAP).  The AMAP also includes useful information about special requirements for organisations wishing to develop education and training programmes, such as minimum qualifications for tutors and assessors, and special resource requirements.